# MODEL

SEN 1702
BLOWING CONVEYOR OVEN

(DIGITAL SCREEN)

# USER & MAINTENANCE MANUAL

ŞENGÜN MAKİNA İNŞAAT ve KIRTASİYE SAN. TİC. LTD. ŞTİ

# These instructions apply only if the country code is shown on the device. If the country code does not appear on the device, refer to the technical instructions that give instructions for adapting the device to the country.

# WARRANTY CERTIFICATE

SERIAL NUMBER             :

SALE DATE     :

Sold at the above date SEN 1702 MODEL BLOWING CONVEYOR OVEN is guaranteed against production failures for 1 Year by ŞENGÜN MAKİNA İNŞAAT ve KIRTASİYE SAN. TİC. LTD. ŞTİ.

# Warranty Terms;

-  Warranty terms start after the product delivery for 1 year.

- Recommended daily working time is 10 hours. The scope of warranty is assessed by year or by working hours.

- Documents are invalid, if haven’t sale date and confirm signature of company. You have to show the warranty certificate to benefit from the warranty.

- Device is guaranteed against defects if mount and use as described ın the use and maintenance manual.

- Detection the location of defects, and elimination of the problems are responsibilities of the ŞENGUN MAKİNA…

- Warranty terms includes only one device. You can’t claim any rights or compensation for another devices.

- If Changes are made at the type, model and serial numbers, the warranty eliminates.

- If material, worker or assembly defects appear, repaired free by ŞENGÜN MAKİNA at the product warranty period (Transportation cost is not included).

- Failures are only repaired by ŞENGÜN MAKİNA’s personal. The intervention of unauthorized person, cancel the appliance of warranty.

- Installation have to be done by ŞENGÜN MAKİNA’s personal. İf usage place changes, customer must give information to ŞENGÜN MAKİNA

- In the event of failure service must be informed.

- The failures and damages given by external factors are out of the warranty terms.

- Any failure of the product warranty period is added to the product warranty time. The repair time is until 20 working days.

- Istanbul courts are competent in case of dispute.

AUTHORIZED PERSON

STAMP –SIGNATURE

PRESENTATION

Dear Users,

Thank you for your prefer our machine and confidence in our company. Our machines are used in 39 countries.

Our machine is manufactured according to international standards. Please read the operating manual to get the mast out of your machine .Please note the following warnings before calling our service.

ŞENGÜN MAKİNA

- Please read the manual carefully before installing the machine.

- The machine will be out of warranty when your operate the machine with at read.

- Please read carefully the manual that gives you information on the installation is prepared in accordance with the legal regulations before arriving at the machine installation.

- For information you coo not understand , ask your nearest out horized service

- Please note that if the technical service personnel is kept waiting you will be billed for each hour.

IMPORTANT WARNINGS AND SAFETY INFORMATIONS

WARNING: This manual contains important safety and installation-operation instructions. Require all operators to read this manual thoroughly before installing, operating or servicing this equipment. Improper installation, adjustment, alteration, service or maintenance can cause property damage, injury or death.

WARNING: This appliance is not intended for use by young children or infirm persons unless they have been adequately supervised by a responsible person to ensure that they can use the appliance safely. Young children should be supervised to ensure that they do not play with the appliance.

WARNING: Improper installation, adjustment, alteration, service or maintenance can cause property damage, injury or death. Read the installation, operating and maintenance instructions thoroughly before installing or servicing this equipment.

WARNING! Do not work around conveyor belt with long hair, loose clothing, or dangling jewelry. Getting caught in the belt could result in serious injury.

WARNING! For your safety, do not store or use gasoline or other flammable vapors or liquids in the vicinity of this or any other appliance.

WARNING! If the power supply cord damaged, do not attempt to operate the unit. Contact a service agent or qualified electrician to repair!

WARNING! Do not use paper when placing food product through the oven! Usage of flammable materials may cause a fire and should never be placed in the oven.

WARNING!       Do not put any object to anywhere of the oven except the conveyor.

WARNING!       Always be careful while you are working.

- The oven must be placed under a ventilator to provide for adequate air supply.

- Minimum clearances must be provided from all walls and combustible materials. Look “spacing requirements” section for more information.

- Always keep the oven clean and clear combustible materials.

- Keep adequate clearance for air openings.

- Do not obstruct the ventilation holes.

- The oven is to be operated only on as shown on the specification plate.

- This manual should be retained for future reference.

- Prepare a list of what you should do when you smell gas and keep it near the oven.

- Gas does not pass through the burner without electricity.

Responsibilities of the Purchaser Company

Responsibilities of the purchaser company are written at bottom;

- To see that the electric and gas services for the oven are installed in accordance with the manufacturers specification.

- İnstalling the oven and placing it as suggested at the catalogue.

- To see that the electric services are connected properly by a qualified installer of your choice.

WARNING

Cleaning and maintenance are the most important factors of oven performance. The processes which are not available in the manual can cancel the appliance of warranty, cause property damage, injury or death. Read and understand the instructions in the manual

CONTENTS

WARRANTY CERTIFICATE

PRESENTATION

SECTION 1: INTRODUCTION

1.1 Introduction of Body Components

1.2 Functions of Components

1.2.1 Oven Body

1.3 Overlap Assembly

1.4 Microprocessor Control

1.5 Cooking Function

1.6 Conveyor (Belt)

1.7 General Information

1.8 Oven Specifications

1.9 Electrical and Technical Specifications Table

1.10 Dimensions

BÖLÜM 2 : ASSEMBLE

2.1 Product Out Sheet

2.2 Forward/Back Waste Collecting Sheet

2.3 Entry and Exit Cover Sheet

2.4 Stopper Sheet (Optional)

2.5 Belt Assembly

2.6 Belt Wire Assembly

2.7 Lower Chassis Assembly (High)

2.8 Lower Chassis Assembly (Low)

2.9 Chassis-Oven Assembly

2.10 Double Floor Oven Assembly

2.11 Air Duct Disassembly / Assembly

SECTION 3: INSTRUCTIONS FOR USE

# 3.1 Distance Requirements

3.2 Ventilation

3.3 General Information of Ventilation

# 3.4 Ventilation Advice

# 3.5 Gas Connection

# 3.6 Installation Elements

# 3.7 SEN 1702 Technical Specifications Table

# SECTION 4: PROGRAMMING-START & STOP

# 4.1 Warnings

# 4.2 Closing

# 4.3 Fault / Warnings Messages

# 4.4 Burner Fault

# 4.5 Functions

# 4.5.1 Temperature Cutting Switch

# 4.5.2 Electrical Installations

# 4.5.3 Limitation Requirements – Gas Ovens

# 4.5.4 Operator Maintenance

# 4.5.5 Cleaning Instructions

# 4.5.6 Protective Maintenance

# SECTION 5: COOKED PRODUCTS

# 5.1 Some of The Cooked Products

# 5.2 Cooking Time and Temperature

# 5.3 Total Cooking Calculation
5.4 Sample Capacity Table

# SECTION 6: SPARE PARTS MANUAL

# SECTION 7: ELECTRICAL DIAGRAM

# SECTION 8: TROUBLESHOOTING

# 8.1 Products Do Not Cooked

# 8.2 Contact With Technical Services

# SECTION 9: LABELS

# NOTE

# SECTION 1: INTRODUCTION

# 1.1 Introduction of Body Components:

NOTE: Entrance of product depends for the cooking style. The out sheet can be attached other side of the oven.

1.2 Functions of Parts:

1.2.1 Oven Body:

Every SEN series oven has a main body.

1.3 Overlap Assembly:

SEN series ovens can be used to overlap two or three. The ovens assembled to the chassis and connect each other.

1.4 Microprocessor Control:

Every single oven is controlled by separate programmable microprocessor. The microprocessor controls all functions of the oven. Oven temperature can be set between 150°C to 300°C. And the cooking time can be set between 1 - 30 minutes. Standby Mode provides energy saving. Also service module helps to the technicians. You can cook standard and delicious products in a short time.

1.5 Cooking Function:

CONVEYOR BELT

AIR DUCT

The oven cooks the products standard and delicious by circulating compressed air.

WINDOW

AIR DUCT

1.6 Conveyor (Belt):

Conveyors provide the input and output of the product. Belt is controlled by microprocessor. Cooking time can be set between 1 - 30 minutes. The speed of belt determines the Products cooking time. The width of belt is 610 mm and consist by interlocking wires.

1.7 General Informations:

- Informations which are located at the following pages will guide you at installation of the oven.

- First of all check the parts of oven carefully.

- Before the installation determine a safety place for oven.

1.8 Oven Specifications:

|  | SEN 1702 |
| --- | --- |
| Conveyor Belt Width : | 610 mm |
| Heating Chamber Length : | 912 mm |
| Heating Chamber Size : | L 912 mm x W 655 mm x H 87 mm |
| Conveyor Cooking Area : | 0,56 m² |
| Overall Dimensions for Single Oven : | Oven Width: 1218 mm Oven Height: 570 mm Total Long: 1957 mm |
| Overall Dimensions for Double Oven : | L 1957 x W 1218 x H 1405,5 |
| Chassis Dimensions of the Single Oven - Weight : | L 1026 mm x W 921 mm x H 559 mm – 24 kg |
| Chassis Dimensions of Double / Triple Oven - Weight : | L 1026 mm x W 921 mm x H 265,5 mm – 16 kg |
| Weight : | 216 kg |
| Transportation Single Oven Weight – Domestic (Machine + Chassis + Packing) : | 295 kg |
| Transportation Single Oven Weight – Abroad (Machine + Chassis + Packing) : | 286 kg |
| Transportation Size – Domestic : | L 1780 mm x W 1340 mm x H 8200 mm |
| Transportation Size – Abroad : | L 1710 mm x W 1270 mm x H 800 mm |
| Average Gas Usage : | 1,3 m³ / hour |
| Usage of gas : | 1,586 m³ / hour |
| Capacity : | 15 kW |
| Allowable Temperature Range : | 150 °C – 300 °C |
| Conveyor Drive System : | 208 - 300 Volt Microprocessor Conveyor control system. |
| Cooking Time : | 1 min – 30 min Adjustable |
| Insulation : | 4 Side |
| Heat Source : | 1 Burner |
| Oven Chamber : | Steel (Galvanized) |
| Steel Enclosure : | 430 Stainless Steel |
| DB Level : | < 70 dba |
| Source : | Electricity and Gas |

NOTE: Burner does not heat the oven every moment. Because Oven Protects chamber temperature. If the chamber temperature is enough, burner shuts down.

1.9 Elektrical and Technical Specifications Table:

Electrical and Technical Specifications Table

| Voltage | Faze | N | T (PE) | Ampere Loading | Gas Inlet Pressure | Capacity (Kw) | Full Performance Gas Usage | Average Gas Usage | BTU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 230V 50/60HZ | 1 | 1 | 1 | 3 | 21 mbar | 15 kW | 1,586 m³/ saat | 1,3 m³ /saat | 75,000 /saat |

Power Supply: Mono FAZE, 230 V AC, 50/60HZ / 3 Amp: 1 Neutral, 1 Grounding
IMPORTANT: Electrical input connection must be connected according to the order of the input terminals.
(PHASE TO PHASE, NEUTRAL TO NEUTRAL, – GROUNDING TO GROUNDING)

NOTE: Specifications can be changed by Şengün Makina. Above Specifications are given for information.

1.10 Dimensions:

Duplex

Single Oven 
(Isometric)

Dimensions Outside

GAS-ELECTRICITY ENTRANCE DIMENSIONS

ELECTRICITY

GAS

FRONT VIEW

GAS INPUT

TOP VIEW

# SECTION 2: ASSEMBLE

OPTIONAL

| NO | PART NAME | UNIT |
| --- | --- | --- |
| 1 | Product Out Sheet | 1 |
| 2 | Forward/Back Waste Collecting Sheet | 2 |
| 3 | Entry and Exit Cover Sheet | 2 |
| 4 | Stopper Sheet (Optional) | 1 |

2.1 Product Out Sheet:

Attach the product out sheet to the conveyor chassis as shown in the figure. And Push it.

2.2 Forward/Back Waste Collecting Sheet:

Front and back waste collecting sheets are taken out as shown in pictures.

2.3 Entry and Exit Cover Sheet:

Covers are set as desired through holes which are different levels

Covers are attached by bakelite bolts.

2.4 Stopper Sheet (Optional):

Stopper, blocks the products and keep them into the belt. Stopper is attached to the belt.

2.5 Belt Assembly:

The conveyor is mounted inside the machine as shown in the picture.

The drive shaft is centered on the clutch pipe.

Clutch pipe is attached by the drive shaft spring as shown in the figure.

Belt assembly is completed. For disassembly, use every steps in reverse order.

2.6 Belt Wire Assembly:

2. Attached by pliers (Middle connecting wire)

1. Installed as shown in the figure. (Edge connecting wire left and right)

4. The final wire assembly

3. If belt wire problem appears, remove a wire as shown in the figure.

For disassembly, use every steps in reverse order.

NOTE: Clean the belt wire once a week or month for better performance.

2.7 Lower Chassis Assembly (High):

00045M HIGH CHASSIS (L 1026 mm x W 921 mm x H 559 mm) SPARE PARTS LIST

| ITEM | SPARE PART NUMBER | SPARE PARTS DESCRIPTION | UNIT |  | ITEM | SPARE PART NUMBER | SPARE PARTS DESCRIPTION | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 07135 | CHASSIS LEG SHEET | 4 |  | 9 | 01957 | BOLT | 18 |
| 2 | 07118 | BRACKET SHEET | 2 |  | 10 | 01961 | WASHER | 18 |
| 3 | 07122 | BRACKET SHEET | 2 |  | 11 | 07120 | BRACKET SHEET | 1 |
| 4 | 07119 | BRACKET SHEET | 2 |  | 12 | 00051 | WASHER | 2 |
| 5 | 01958 | BOLT | 32 |  | 13 | 00048 | BOLT | 2 |
| 6 | 01960 | WASHER | 32 |  | 14 | 01964 | WHEEL | 4 |
| 7 | 01965 | NUT | 32 |  | 15 | 01963 | WASHER | 16 |
| 8 | 01959 | NUT | 18 |  | 16 | 00047M | BRACKET SHEET GROUP | 2 |

2.8 Lower Chassis Assembly (Low):

00046M LOW CHASSIS (L 1026 mm x W 921 mm x H 265,5 mm) SPARE PARTS LIST

| ITEM | SPARE PART NUMBER | SPARE PARTS DESCRIPTION | UNIT |  | ITEM | SPARE PART NUMBER | SPARE PARTS DESCRIPTION | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 07123 | CHASSIS LEG SHEET | 4 |  | 8 | 01960 | WASHER | 16 |
| 2 | 07125 | BRACKET SHEET | 2 |  | 9 | 01965 | NUT | 16 |
| 3 | 01958 | BOLT | 16 |  | 10 | 00051 | WASHER | 2 |
| 4 | 07126 | BRACKET SHEET | 1 |  | 11 | 00048 | BOLT | 2 |
| 5 | 01957 | BOLT | 18 |  | 12 | 01964 | WHEEL | 4 |
| 6 | 01961 | WASHER | 18 |  | 13 | 01963 | WASHER | 16 |
| 7 | 01959 | NUT | 18 |  | 14 | 00048M | BRACKET SHEET | 2 |

2.9 Chassis-Oven Assembly:

2.10 Double Floor Oven Assembly:

Note: Bolts are included in the package

| ITEM | SPARE PART NUMBER | SPARE PART DESCRIPTION | UNIT |
| --- | --- | --- | --- |
| 1 | 02056 | HOLDER | 4 |
| 2 | 00046 | BOLT | 8 |

CAUTION!  Oven must to be cold.

- Remove product output plate and product stopper plate. (See Pg. 16)

2. Remove conveyor waste collection plates. (See Pg. 15)

3. Remove conveyor belt. (See Pg. 17-18)

4. Remove cooking room covers. (See Pg. 16)

5. Remove air canals as shown figure 1 pull in the direction of arrow.

6. You need to write codes for re-assembly like used to be. 

Marking Examples;
Upper air canals: U1, U2, U3, U4
Bottom air canals: A1, A2, A3, A4

7. Remove air canals as shown Fig. 2 and Fig. 3. Apply marking when you remove air canals.

Marking Examples;
Hood U1,
Internal Plate U1,
External Plate U1,

8. You can do cleaning after this process. You can do assembly do reverse what you did.

2.11 Air Duct Disassembly / Assembly:

Figure 1

Figure 2

Fume Hood

Inner Plate

Outer Plate

SECTION 3: INSTRUCTIONS FOR USE

3.1 Distance Requirements:

The oven must have 130 mm of clearance from combustible surfaces. In case other equipment is located on the right side of oven, a minimum clearance of 620 mm is required from that equipment.

FOR ALL OVENS: A 620 mm clearance at the rear of the oven must be obtainable for service access.

FOR FIXED OVENS ANY TIME: A permanently installed (unmovable) oven requires a minimum of 340 mm clearance on the right-hand side to allow for conveyor removal, cleaning, and servicing.

NOTE: Do not install this (these) oven(s) in any area with an ambient temperature in excess of 40° C. Doing so will cause damage to the unit.

3.2 Ventilation:

Ventilation is very important especially gas ovens. Determine your ventilation system according to your country fire and safety rules. The information to guide you about the ventilation is in the next section.

3.3 General Information About Ventilation:

Ventilation channel or hood should be sufficient to attract heat and cooking odor. In addition, ventilation in gas ovens must be sufficient to expel combustion products. Operation and properly configured ventilation is the responsibility of the furnace owner.

The ventilation mouth (hood) should be in harmony with the general ventilation and heating system.

ANTENTİON: Do not allow air to flow through the cooking tunnel. The air must not be directed to the front top of the oven, to the side or to the rear.

3.4 Ventilation Advice:

SMOKE CATCH AREA

SMOKE CATCH AREA

258mm

258mm

You are ready for services to be connected. This should be done by a qualified electrician, or installer of your choice.

3.5 Gas Connection:

In order to have a safe and efficient furnace, the oven must be installed properly. Installation of gas connection must be done properly. National gas Law rules are valid.

- When the furnace is subjected to a pressure test of more than 35 mbar, the opening-closing valve of the furnace and the gas supply pipe must be separated from the gas supply pipe.,

- The gas supply pipe should be isolated from the oven by the opening - closing valve on the gas supply pipe during pressure tests equal to or less than 35 mbar.

DO NOT ATTEMPT TO OPERATE THE OVEN until connection of utility service and installation has been fully inspected (START-UP CHECKOUT) by and Authorized Service Technician.This is important service, In order to assure the oven(s) is/are properly installed and in working order. The warranty becomes effective upon verification of proper installation.

The warranty shall not apply if the oven(s) is/are started up and operated prior to the “START-UP CHECKOUT” being performed by an Authorized Service Technician.

DANGER!  Do not work around conveyor belt with long hair, loose clothing, or dangling

jewelry. Getting caught in the belt could result in serious injury.

DANGER!  If the power supply cord appears to be damaged, do not attempt to operate

the unit. Contact a service agent or qualified electrician to repair!

3.6 Installation Elements:

1 x 10 A (K AUTOMAT) – Fuse

Cable section: 1, 5 mm² (TTR Cable – 1 Faze / 1 Neutral / 1 Ground)

NOTE: TTR Cable is Flexible, shatterproof and loosen proof.

Grounding has to be proper.

Use regulator if voltage is high or low.

NOTE: If you fallow the above steps, assembly process can be made smoothly and quickly. We wish you good work.

3.7 SEN 1702 Technical Specifications Table:

Oven Electricity – Gas Technical Specifications Table

| Voltage | Faze | N | T (PE) | Ampere Loading | Gas Inlet Pressure | Capacity (Kw) | Full Performance Gas Usage | Average Gas Usage | BTU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 230V 50/60HZ | 1 | 1 | 1 | 3 | 21 mbar | 15 kW | 1,586 m³/ hour | 1,3 m³ /hour | 75,000 /hour |

Power Supply: Mono FAZ, 230 V AC, 50/60HZ / 3 Amp: 1 Neutral, 1 Ground
IMPORTANT: Electrical input connection must be connected according to the order of the input terminals.
(PHASE TO PHASE, NEUTRAL TO NEUTRAL, – GROUNDING TO GROUNDING)

Terminal
Blog

1

2

3

PE

N

L1

GAS

ENTRANCE

POTENTIAL GROUNDING

FUSE

POWER CABLE ENTRANCE

SECTION 4 : PROGRAMMING , OPERATING AND CLOSING

All functiens of the oven are controlled by the microprocessor. The oven must be pre-programmed for operation. The following pages are described step by in programming.

OVEN CONTROL DEVICE

Working Condition

Working Temperature : -20 - 70°C

Maximum Damp : %90 Rh (without condensation)

Height : Up to 2000 meters

Environments where device use is not suitable.

Corrosive atmospheric environments.

Explosive atmospheric environments.

Home applications. (The device only can be used industrial applications.)

Feeding Voltage İnput

OVEN CONTROL

Termocouples  iiinputİnput

Driver Fault

Termocouples J,K

Heater Output

Failure İnput

Engine Driver Outlet

Start Relay

START OUTPUT

Burner Fault

230 Vac

(For Gaseous Model)

Thermocouple

İnput

Heater Output

SSR Output

TEMPERATURE DISPLAY

ENGINE DRIVE TİME VALUE ADJUSTMENT

HEATER TEMPERATURE

VALUE ADJUSMENT

1 - 2. Temperature Set Increase and Decrease Buttons : With these buttons, the temperature set volves of program number that appear on the upper display are changed.

3 - 4. Engıne Driver Time Increase and Decrease Buttons : Set the time in minutes and seconds with these buttons.

5 - 6. Program Increase and Decrease Buttons: The desired program number is selected using these buttons.

7. Engine Start Button : Press this button to start the engıne.

8.Engine Stop Button: Press this button to stop the engıne.

9.Heater Start Button :Press this button to activate the heater output is active.

10. Heater Stop Button: This button stops the output when heater output is active.

11. Belt Back Button: Rotates the engıne counterclockwise when this button is pressed.

12. Belt Forwed Button : Rotates the engine clockwise when this button is pressed.

13. Program Set Button : Press this button once for 5 seconds and enter the set section.

14. ON/OFF Button : With this button the device is returred to the “on’’ or “off ’’ state . Button turns off when the device is switehed off screen turns off.

The values of that program appear on the screens after the program number has been entered. Set values are changed with the help of boost and decrease buttons. Press ‘P’ button for 5 seconds to enter the set section . In this cose the changed set values are recorded. The set values that are modified are instantaneous values. These values are not saved to the program. The old values of the program are displayed again when switching to the off position or when the power is turned off.

Programs are only saved when the heater is in the stop position. The set values changed in the start position are instantaneous values. These values are not saved to the program . The old values of the program are displayed again when switching to the stop position.

4.1 Warnings:

1. It the product does not cook, check the cooking times and temperaturenstting conveyor can be spinning fast..

2. It the product is over  cooked , check the cooking times and temperature settings can be turning slowly.

3. It all values are normal nut the product still does not cook or over heat , call the manufacturer.

4. This is sue is fixed by pressing the ON/OFF  button on the display when the burnor  in the gas ovens gives a fault alarm. The more detailed description is on page 57.

5. Do not keep combustible and explosive material around the machine.

4.2 Closing 
To turn off the oven, touch ON/OFF  and switch off with 0-1 switch .

4.3 Fault / Warning Messages :

Our machines has a checked fault warnings message. Ann unexpected error in the operation of the oven that may ocur will appear on the as it appears below.

FAULT 1: Fault 1 code will be observed if no flame is generated during ignition. It can go with reset.

FAULT 2: This error code will occur if the flame is observed without heat demand. It can go with reset.

FAULT 3: The current error for the limit thermostat switch, this error will be observed depending on the limit thermostat and reset is required to remedy.

FAULT 5: The air pressure switch (prosestat) is the error code caused by failure. The error will be observed if the pressure switch contact does not come to the appropriate position within 10 sec with the fan activating.

FAULT 8: Under normal operating conditions, the ionization perception is continuously controlled by the card. This error will be observed if there is a negative in this control.

FAULT 9: In normal operating conditions, the gas valve is constantly checked by the card. This error will be observed if there is a negative in this control.

FAULT 13: You will not see this error code because the reset will be done through the feed.

FAULT 21: It will occur due to a negativity in the card itself.

FAULT 25: CRC error – matching error on CRC codes (diferent software versions ).

FAULT 23: The wait times specified in the burner startup entries do not match after the on-hold period.

FAULT 26 : Pressing the button for more than 10 seconds causes an error until the reset button is released.

4.4 Burner Fault:

Occurs when the burner does not fire or if there is fault in the burner system.

4.5 Functions:

4.5.1 Temperature Cutting Switch :

Our  oven has a temperature cut. Off switch for you sefety and protection . This switch is specially designed to prevent the oven from over heating and causing damage in aa small posibility , if the operating temperature of the furnece increases and the power is prevented from being transferred to the furnaceç.No parts are damaged due to excessivve temperature.

WARNING! Do not use an external switch to do not block to the temperature switch.

4.5.2 Electrical Installations:

WARNING! Grounding must be done properly before install of the oven. Electrical damage causes fatal injuries or death.

When installation is made, the product must be fully grounded.

For All Foreign Countries: Apply local electrical rules.

- All Poles connect switches have 3mm wide.

- To prevent electrical shock, the potential connection grounding cable hood is located on the back of the oven. This allows the oven to be combined into an external connection system.

- If you use more than one oven (double or triple) each oven must has its own switch and all switches should be close together.

4.5.3 Limitation Requirements – Gas Ovens:

- The installation shall be carried out by a fast-connecting device in accordance with the standards for fasteners for gas-operated devices with a gas connector in accordance with the fastener standards for movable gas appliance.

- The installation of the restriction should limit the movement of the oven, regardless of the connection element. The quick disconnect tool or its pipe should limit the movement of the oven.

- If the restriction is removed due to cleaning or maintenance, it must be returned back to the original after cleaning or maintenance has finished.

- Connect the “B” part and its cable to “A” part with bolt.

- Connect the “C” part and its cable to support of the wall “D” or “E” with bolt.

NOTE: Connection points are the same with double or triple oven assembly.

4.5.4 Operator Maintenance:

WARNING!  Disconnect power supply before servicing or cleaning this oven. Safeguard power so it cannot be accidentally restored. Failure to do so could result in dismemberment, electrocution, or fatal injury. There is more than one power supply connection point when ovens are stacked.

As a result of extensive engineering work, the oven has been designed for easy and convenient maintenance. No lubrication is required for this furnace.

To use the oven in the most efficient way, it is essential to clean the oven. See below for cleaning instructions. Frequency is determined by factory recommendations. Your use and type of product will determine your frequency of cleaning.

If the oven fails to operate, check the circuit breaker to be sure it is turned on. Also, before calling technical service, check that the power cut switch on the control panel is on. Technical service information must be located on the oven or contact the factory to find out the nearest technical service.

4.5.5 Cleaning Instructions:

The oven contains electrical components. Before cleaning the oven, switch off and disconnect the oven from the electrical supply.

No electrical components should be subjected to moisture. It is, therefore, important that the oven is wiped down carefully. NEVER throw buckets of water over the oven or subject it to pressure washing from a hose or a pressure spray. If water or other liquid is spilled on the oven, make sure that none has entered the control box area before switching on. If in doubt, call your service company.

WARNING!  Oven must be cool. Do not use power-cleaning equipment, steel wool, or wire                                
                            brushes on stainless steel surfaces.

DAILY;

- Clean the outer surfaces with a mild detergent cloth. You can also use a commercial stainless steel cleaner product. Then clean the detergent residue with a wet cloth.

- Wash the preservatives of the front and rear waste collection plates with a mild detergent and rinse with water.

- Wash the outlet sheet with a mild detergent and rinse with water.

- Wash the stoppers with a mild detergent and rinse with water.

- Clean the loose particles inside, then wipe them with a mild detergent, and clean the detergent residue with a wet cloth.

- Clean the conveyor with a clean cloth or with a soft wire brush.

On exterior of oven, deposits of baked-on splatter, oil, grease, or light discoloration may be removed with any of several commercial cleaners. Consult with your local supplier.

MONTHLY;

- Remove, disassemble and clean air vents. Refer to "Air Duct Removal ”for more information.

- Remove the tape, dismantle and clean it. See “tape mounting and wire mounting” for more information.

- Remove, disassemble and clean the oven room front and rear covers.

NOTE: Make sure that the hood is cleaned according to the specifications of the ventilation equipment vendor.

4.5.6 Protective Maintenance:

Although this oven has been designed to be as trouble free as possible, periodic Preventive Maintenance is essential to maintain peak performance. It is necessary to keep the motors, fans, and electronic controls free of dirt, dust and debris to insure proper cooling. Overheating is detrimental to the life of all components mentioned.

The periodic intervals for preventative cleaning may vary greatly depending upon the environment in which the oven is operating.

You must discuss the need for Preventive Maintenance with your Authorized Service Company to establish a proper program.

SECTION 5: COOKED PRODUCTS

5.1 Some of the Cooked Products

- Brownie

- Hamburger

- Hot Dogs

- Omelet

- Pancakes

- Mixed Vegetable Plate

- Steakhouse

- Skewers

- Meatball

- Potato

- Borek

- Chicken Wings

- Grilled Chicken

- Chicken Saute

- Bread

- Breakfast

- Fish

- Seafood

- Pizza

- Lavas

- Lahmacun

- Pita

- Kunefe

- Chocolate Cookie

- Pie

- Etc. Many Foods

Comparing the performance of our furnace with other furnace types especially when baking pizzas, the performance of our SEN furnaces is clearly ahead

5.2 Cooking Time and Temperature:

PİZZA

- 260 °C

- TIME: 6 MINUTES

- ALUMINIUM PAN
- FRESH

- 260 °C

- TIME: 5 MINUTES

- OVER CONVEYOR
- FRESH

PİDE

5.3 Total Cooking Calculation:

Pan Length 
(A)

Sample 1:

Oven Chamber Length: 915 mm
Cooking Time: 2 min
Pan Length: 250 mm

250

1

2

3

2

1

Sample 2:

Oven Chamber Length: 915 mm.
Cooking Time: 5 min.
Pan Length: 500 mm.

500

Pan Length 
(B)

5.4 Sample Capacity Table:

| ÖRNEK KAPASİTE TABLOSU |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pizza Diameter | 3 minutes | 3 ½ | 4 minutes | 4 ½ |
| 30 cm | 120 | 103 | 90 | 80 |
| 36 cm | 70 | 60 | 53 | 68 |
| 41 cm | 50 | 43 | 38 | 33 |
| Pizza Diameter | 5 minutes | 5 ½ | 6 minutes | 6 ½ |
| 30 cm | 72 | 65 | 60 | 55 |
| 36 cm | 54 | 49 | 45 | 42 |
| 41 cm | 30 | 27 | 25 | 23 |
| Pizza Diameter | 7 minutes | 7 ½ | 8 minutes |  |
| 30 cm | 51 | 48 | 45 |
| 36 cm | 39 | 36 | 34 |
| 41 cm | 21 | 20 | 19 |

SECTION 6: SPARE PARTS MANUAL

SPARE

PARTS

MANUAL

M00426 FRONT

| ITEM | SPARE PART NUMBER | SPARE PART DESCRIPTION | UNIT |  | ITEM | SPARE PART NUMBER | SPARE PART DESCRIPTION | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 01815 | LIMIT THERMOSTAT | 1 |  | 20 | 01515M | ENGINE CONNECTION SHAFT | 1 |
| 2 | 01184 | RECORD | 1 |  | 21 | 01878 | LABEL (PRODUCTION INFORMATION) | 1 |
| 3 | 01089 | RECORD NUT | 1 |  | 22 | 01865 | GLASS BRAKER TERMINALS | 1 |
| 4 | 02209 | TERMINALS (GROUNDING) | 1 |  | 23 | 01882 | CONDENSER | 1 |
| 5 | 02210 | TERMINALS (BLUE) | 1 |  | 24 | 02194 | CONDENSER | 1 |
| 6 | 02211 | TERMINALS | 1 |  | 25 | M00369 | SSR BOX | 1 |
| 7 | 00292 | LIMITER | 1 |  | 26 | 01886 | TERMINAL RAIL | 1 |
| 8 | 07038 | SUPPORT SHEET | 1 |  | 27 | 02178 | BOLT | 4 |
| 9 | 01648 | COVER SHEET | 1 |  | 28 | 01857 | CONTROL BOX COVER | 1 |
| 10 | 00046 | BOLT | 15 |  | 29 | 00061 | TERMINAL | 2 |
| 11 | 00042 | WASHER | 13 |  | 30 | 01850 | ENGINE | 1 |
| 12 | 00041 | BOLT | 9 |  | 31 | 01867 | BOLT | 4 |
| 13 | 01103M | AIR PROTESTAT | 1 |  | 32 | 00006 | WASHER | 4 |
| 14 | 02580 | BOLT | 2 |  | 33 | 00317 | WASHER | 4 |
| 15 | 00365M | CONTROL BOX | 1 |  | 34 | 01305 | NUT | 8 |
| 16 | 01879 | KONTAKTOR | 1 |  | 35 | 01862 | STEP ENGINE SHEET | 1 |
| 17 | 01831 | BOLT | 2 |  | 36 | 01861 | ENGINE SHEET | 1 |
| 18 | 00364M | SCREEN | 1 |  | 37 | 02351 | BURNER COMMUNICATIONS CARD | 1 |
| 19 | 08091 | BUTTON | 1 |  | 38 | M00342 | BURNER | 1 |

M00426 (FRONT) SPARE PARTS LIST

07014M FRONT COVER

07014M FRONT COVER SPARE PARTS LIST

| ITEM | SPARE PARTS NUMBER | SPARE PARTS DESCRIPTION | UNIT |
| --- | --- | --- | --- |
| 1 | 01558 | BOLT | 4 |
| 2 | 07026M | WINDOW FRAME | 1 |
| 3 | 07161 | WINDOW EDGE PLATE | 1 |
| 4 | 01548 | WATCHING WINDOW | 1 |
| 5 | 01762 | WINDOW EDGE PLATE | 1 |
| 6 | 07160 | GLASS FRAME CONNECTION | 2 |
| 7 | 01759 | WINDOW EDGE PLATE | 2 |
| 8 | 01752-1 | RUBBER PLUG | 2 |
| 9 | 01753-1 | COVER HANDLE | 1 |
| 10 | 02342 | BOLT | 2 |
| 11 | 00045 | FIX | 6 |
| 12 | 101503 | BOLT | 4 |
| 13 | 04077 | WASHER | 4 |
| 14 | 00157 | BOLT | 2 |
| 15 | 07172 | GLASS OPEN/CLOSE LAMA | 1 |
| 16 | 07170 | GLASS OPEN/CLOSE LAMA | 1 |

M00399 BACK COVER (TYPE-1)

M00399 BACK COVER (TYPE-1) SPARE PARTS LIST

| ITEM | SPARE PARTS NUMBER | SPARE PARTS DESCRIPTION | UNIT |
| --- | --- | --- | --- |
| 1 | M00346 | BODY | 1 |
| 2 | 00458M | BACK ISOLATED COVER | 1 |
| 3 | 02581 | NUT | 3 |
| 4 | 07030 | STUD | 6 |
| 5 | 01959 | NUT | 6 |
| 6 | 01961 | WASHER | 10 |
| 7 | 01963 | WASHER | 6 |
| 8 | 07031 | FAN GUARD SHEET | 1 |
| 9 | 00279 | BOLT | 1 |
| 10 | 01960 | WASHER | 1 |
| 11 | 00277 | WASHER | 1 |
| 12 | 00398M | FAN | 1 |
| 13 | 00556 | BOLT | 3 |
| 14 | 07156 | ENGINE SHEET | 1 |
| 15 | 101511 | F.I.R ENGINE | 1 |
| 16 | 04074 | BOLT | 4 |
| 17 | 05529 | WASHER | 4 |
| 18 | 01672 | BOLT | 2 |
| 19 | 01728 | TERMINALS | 1 |
| 20 | 01317 | REZISTANCE OUTPUT TUBE | 1 |
| 21 | 07004M | BACK COVER | 1 |
| 22 | 01299 | WASHER | 18 |
| 23 | 00042 | WASHER | 18 |
| 24 | 00041 | BOLT | 18 |

M00398 BACK COVER (TYPE-2)

M00398 BACK COVER (TYPE-2) SPARE PARTS LIST

| ITEM | SPARE PARTS NUMBER | SPARE PARTS DESCRIPTION | UNIT |
| --- | --- | --- | --- |
| 1 | M00346 | BODY | 1 |
| 2 | 00458M | BACK ISOLATED COVER | 1 |
| 3 | 07168 | ENGINE SHEET | 1 |
| 4 | 01672 | BOLT | 2 |
| 5 | 01728 | TERMINALS | 1 |
| 6 | 04074 | BOLT | 4 |
| 7 | 01961 | WASHER | 10 |
| 8 | 05529 | WASHER | 4 |
| 9 | 00556 | BOLT | 4 |
| 10 | 01299 | WASHER | 22 |
| 11 | 00042 | WASHER | 22 |
| 12 | 07169 | SHAFT | 4 |
| 13 | 07116 | BAHCIVAN ENGINE | 1 |
| 14 | 07030 | STUD | 6 |
| 15 | 01959 | NUT | 6 |
| 16 | 01963 | WASHER | 6 |
| 17 | 07031 | FAN GUARD SHEET | 1 |
| 18 | 00400M | FAN | 1 |
| 19 | 07004M | BACK COVER | 1 |
| 20 | 00041 | BOLT | 18 |
| 21 | 01317 | REZISTANCE INPUT MACARON | 1 |

M00346 BODY

M00346 (BODY) SPARE PARTS LIST

| ITEM | SPARE PARTS NUMBER | SPARE PARTS DESCRIPTION | UNIT |
| --- | --- | --- | --- |
| 1 | 07021 | AIR DUCT PLASTERING SHEET | 8 |
| 2 | 07017M | INTERNAL AIR DUCT SHEET | 8 |
| 3 | 07054 | FIX SHEET | 8 |
| 4 | 07016M | AIR DUCT SHEET | 8 |
| 5 | 00416M | BODY | 1 |
| 6 | 07000 | TOP/BOTTOM COVER | 2 |
| 7 | 00046 | BOLT | 12 |
| 8 | 00124 | HANDLE | 8 |
| 9 | 00417M | SIDE PACKAGE | 2 |
| 10 | 07010M | SIDE PACKAGE | 2 |
| 11 | 07027M | ENTRANCE/OUT SHEET | 2 |
| 12 | 01731 | THERMOCOUPLES | 1 |
| 13 | 01596-1 | PIN | 4 |
| 14 | 00441M | INTERNAL AIR DUCT SHEET | 2 |

SPARE PARTS

These materials are supplied with the machine. According to the cooking quality of the product to be cooked, air   feeder are removed and these sheets are attached.

M00369 SSR BOX

M00369 (SSR BOX) SPARE PARTS LIST

| ITEM NO. | SPARE PARTS NUMBER | SPARE PARTS DESCRIPTION | UNIT |
| --- | --- | --- | --- |
| 1 | 01295 | BOLT | 7 |
| 2 | 01290 | FAN | 1 |
| 3 | 02320 | SPEED CONTROL | 1 |
| 4 | 00301 | FUSE | 1 |
| 5 | 02334 | AIR ROUTING SHEET | 1 |
| 6 | 02319 | POWER SUPPLY | 1 |
| 7 | 01307 | SPONGE | 2 |
| 8 | 01271 | BOLT | 4 |
| 9 | 02634 | NUT | 11 |
| 10 | 01306 | SPONGE | 2 |
| 11 | 01011 | SSR SHEET | 1 |
| 12 | 00292 | LIMITER | 1 |
| 13 | 01296 | WASHER | 7 |
| 14 | 00293 | TERMINALS (GROUNDING) | 1 |
| 15 | 02208 | FUSE | 1 |
| 16 | 01886 | TERMINALS RAIL | 1 |
| 17 | 00046 | BOLT | 2 |
| 18 | 01822 | SSR RELAY | 1 |
| 19 | 01292 | VARISTOR | 1 |

M00347 BELT

OPTIONAL

M00347 (BELT) SPARE PARTS LIST

| ITEM NO. | SPARE PART NUMBER | SPARE PART DESCRIPTION | UNIT |
| --- | --- | --- | --- |
| 1 | 02013 | BELT DRIVE BEARING | 4 |
| 2 | 07093 | WASTE PAN | 2 |
| 3 | 00096 | PIN | 1 |
| 4 | 07095 | STOPER SHEET | 1 |
| 5 | 07000M | WIRE BELT | 1 |
| 6 | 07029 | PRODUCT OUT SHEET | 1 |
| 7 | 00374M | CONVEYOR CHASSIS | 1 |
| 8 | 00319M | BELT DRIVE SHAFT | 1 |
| 9 | 00318M | BELT IDLE SHAFT | 1 |

M00342 BURNER

| ITEM | SPARE PART NUMBER | SPARE PART DESCRIPTION | UNIT |  | ITEM | SPARE PART NUMBER | SPARE PART DESCRIPTION | UNIT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 01666 | BURNER ENGINE | 1 |  | 20 | 00042 | WASHER | 4 |
| 2 | 01698 | ENGINE CONDENSER SHEET | 1 |  | 21 | 07076 | NOZZLE | 1 |
| 3 | 01685 | WASHER | 8 |  | 22 | 01504M | GAS ENTRANCE PIPE | 1 |
| 4 | 00048 | BOLT | 4 |  | 23 | 07022M | BURNER PIPE | 1 |
| 5 | 02581 | NUT | 1 |  | 24 | 01518M | IGNITION ELECTRODE | 1 |
| 6 | 02541 | NUT | 3 |  | 25 | 07023M | BURNER CORE | 1 |
| 7 | 01699 | AIR PROCESS PIPE | 1 |  | 26 | 07025M | BURNER ENTRANCE PIPE | 1 |
| 8 | 07075 | NOZZLE | 1 |  | 27 | 01505M | AIR SET CHASSIS | 1 |
| 9 | 01671 | TEST NOZZLE | 1 |  | 28 | 01506M | AIR CHASSIS COVER | 1 |
| 10 | 01686 | GAS VALVE | 1 |  | 29 | 07024M | BURNER BODY | 1 |
| 11 | 02640 | NIPPLE | 3 |  | 30 | 01672 | BOLT | 2 |
| 12 | 07077 | ELBOW | 1 |  | 31 | 01881 | VALVE | 1 |
| 13 | 02649 | ELBOW | 2 |  | 32 | 01880 | BRAIN | 1 |
| 14 | 02639 | RECORD | 1 |  | 33 | 07139 | RECORD | 1 |
| 15 | 01689 | NIPPLE | 1 |  | 34 | 101504 | CYLINDER HEAD GASKET | 2 |
| 16 | 07078 | PIPE | 1 |  | 35 | 101510 | CYLINDER HEAD GASKET | 1 |
| 17 | 01695 | BOLT | 8 |  | 36 | 00395M | ELBOW | 1 |
| 18 | 01670 | WASHER | 1 |  | 37 | 00052 | NUT | 4 |
| 19 | 00041 | BOLT | 4 |  |  |  |  |  |

M00342 BURNER SPARE PARTS LIST

SECTION 7: ELECTRICAL DIAGRAM

SECTION 8: TROUBLESHOOTING

8.1 Products Do Not Cooked:

Products are not cooking

Appears on the screen ready. Fault fixed.

Check cooking time and heat settings. Conveyor can be turning fast.

High temperature cut-off switch can be active.

Burner can be failure.

Press the red button on the high – temperature cut-off switch.

Turn ON/OFF   the 0-1 switch several times on the front of the electrical box.

Motor may be damaged, change.

If some error still persists in the oven, call technical service.

8.2 Contact with Technical Services:

If the oven fails to operate, check the circuit breaker to be sure it is turned on. In addition, be sure that the circuit breaker is good before you call the Authorized Service Agency. The name and phone number of the Authorized Service Agency should be located on the oven or contact the factory for the name of the nearest Authorized Service Agency.

BÖLÜM 9: LABELS

(High Temperature)	                               (Grounding)

Electrical Input

Warning Labels

Technical Label (Product Card)

Warning Labels

NOTE:

ŞENGÜN MAKİNA İNŞAAT VE KIRTASİYE SAN. TİC. LTD. ŞTİ.

İkitelli Organize Sanayi Bölgesi, Atatürk Bulvarı, Pufa İş Merkezi No: 98 Bodrum Kat: 2-3

Başakşehir/ İstanbul/ Türkiye

Phone: +90 212 671 23 23 (Pbx)

Phone 2: +90 212 671 84 84

Fax: +90 212 671 88 28

GSM: +90 533 957 14 53

e-Mail : info@sengunmakina.com

web : www.sengunmakina.com